17A Robertson road

- Relocate radiator from west wall to east side wall below window, replacing the rad with a single 1000x600 to allow room to pass by. Pipework is currently in 15mm copper pipe.

- Install Hit&Miss vent into cupboard door.

- Increase the size of the trickle vents on both casements (currently 260mm)

- Overboard cills with UPVC (north size opening 810mm) (east side opening 1230mm)

- Remove defective plaster/ plasterboard and replace with new including reskim and plasterboard. (Approx. 4sm) I believe that this construction is solid wall so the north wall may be insulated on the internal side.

- Prepare and redecorate complete room, walls, ceiling and woodwork.

- Install key clamp handrail

- Remove all defective and damaged render to both sides.

- Rebuild loose brickwork ( approx. 1 course)

- Prepare and apply render finish.

- Replace coping stones (360x600)

(Neighbouring property side)

- Hack out and repair sections of render to the rear elevation including filling of redundant holes.

- Apply a silicone bead around the perimeter of both windows.

- Power wash down algae and leave ready for decorations.

- Replace fascia

- Replace rainwater goods and fittings

- Remove and set aside coping stones

- Hack off render to Livewest side.

- Apply scratch coat and top coat

- Leave ready for cyclical decorations.