14 carter walk

Kitchen

- Isolate, disconnect and set aside extractor fan.

- Hack off plaster back to blockwork to edge of window revel. (approx. 6sm)

- Install helifix bars through mortar joints and overlay blockwork with a brick reinforming mesh to help support thermal movement.

- Dub out and apply 2 coats of plaster finish.

- Prepare hairline movement cracks to wall and ceiling lines with a flexible decorators sealant.

- Decorate all walls to match best as possible. https://www.dulux.co.uk/en/colour-details/lime-zest-1

- Rake and reseal window board.

- Reinstate extractor fan.

Approx. room size 30sm.

Lounge

- Hack plaster back to blockwork on half of back wall. (approx. 10sm)

- Install helifix bars and brick reinforming mesh to mortar and blockwork around the damaged sections.

- Dub out and apply 2 coats of plaster.

- Apply 1 coat of mistcoat and 2 top coats of magnolia

- Prepare crack to ceiling by stairs and touch in decorations.

Stairs and landing (approx. 12sm)

- Replace loft hatch

- Re-fix plasterboard back to joists

- Prepare and smooth out repair joints.

- Decorate ceiling in white emulsion.

Front Bedroom

- Disconnect radiator and set to one side.

- Hack off plaster back to blockwork to window wall and R/H wall. (approx. 24sm)

- Install helifix bars and brick reinforming mesh to mortar and blockwork around the damaged sections.

- Dub out both walls and apply 2 coats of multi finish plaster.

- Apply mistcoat to fresh plaster and 2 coats of magnolia.

- Reinstate radiator

Back Bedroom

- Disconnect radiator and set to one side

- Hack off plaster back to blockwork to window wall and R/H wall. (Approx. 24sm)

- Install helifix bars and brick reinforming mesh to mortar and blockwork around the damaged sections.

- Dub out both walls and apply 2 coats of multi finish plaster.

- Apply mistcoat to fresh plaster and 2 coats of magnolia.

- Reinstate radiator